
ACTION PLANNED IN PERIOD 2
INFORMATION SHORT CUT
introduction of fax, electonic data interchange (EDI) to the customer, supplier and within the company
results:

- shorter information flow time
- higher planning accuracy
- shortening throughput time
costs:


500 EUR for the investment in hardware
logistic rule  I:
cutting down information flow time is cheaper than cutting down  material flow time
logistic rule II:
5 minutes of information flow time is less capital intensiv  than 5 minutes of material  flow time.
SHORTENING  TRANSPORT CYCLES
all deliveries now possible in every turn
results:
- shorter replace cycles for the sales- order processing

- cutdown of the inventories in plant FP- and Salesstock 
costs:
- 800 EUR for additional transport vehicle
LOT SIZE REDUCTION
The minimum lot size in the plant - final assembly  is reduced from 5 to 3 units
results:
- shorter replace cycles for the plant - FP-stock

- cutdown of the inventories in plant FP- stock
costs:
- increase of  production control costs

ACTION PLANNED IN PERIOD 3
WITHDRAWAL OF WAREHOUSES
black device delivery directly from the plant.
results:

- cut down internal transportation costs
- cutdown of the inventories in Sales-stock
- shortening throughput time
caution !
disharmony in the in the planning system could be              caused by lower inventory level. 
LOT SIZE REDUCTION 
The minimum lot size in the plant - final assembly  is reduced from 3 to 1 unit.
Black  and white devices are loadable on any machinery in the plant-subassembly.
results:
- shorter replace cycles for the plant - FP-stock

- cutdown of the inventories in plant FP- stock

- flexible response to customer orders
costs:
- 500 EUR for new lot preparing machinery
Suppliercertification and JIT-delivery
Preselection of low quality supplier with the help of supplier-audits-systems
JIT- delivery-contract for the red units.
prerequisites:
- decreasing the number of supplier

- quality contracts with supplier 

- demand is generated by the assembly line
results:
- 100% quality on time

- shorter replace cycle

- cutdown of the inventories in plant material - stock

- relief for the inspection capacity
costs:
- 300 EUR for supplier audits and contracts.
FLEXIBILITY IN TRANSPORT
new contracts with shipping companies
results:
- no transport capacity limits 

- improved  service for customer requested dates
costs:
- 50 EUR for contracts.
Copyright: RiegerTraining 2017

